# NETWORKWEAVER SYSTEM - COMPLETE SETUP GUIDE FROM SCRATCH

Document: Comprehensive Installation & Configuration Guide

Date: February 7, 2026

System: NetworkWeaver
MikroTik Network Automation  Monitoring Platform

TABLE OF CONTENTS

1. Prerequisites & Software Requirements

2. Setting Up VMware CHR Router (192.168.247.2 on VMnet8 NAT)

3. Configuring RouterOS SNMP with Community 'public'

4. Setting Up Docker Containers (Prometheus & SNMP Exporter)

5. Setting Up the FastAPI Backend (Port 8000)

6. Setting Up the React+Vite rontend Port 5173)

SECTION 1: PREREQUISITES & SOFTWARE REQUIREMENTS

Before setting up NetworkWeaver, ensure the following software is installed:

1.1 REQUIRED SOFTWARE

a) VMware Workstation Pro

- Purpose: Hosts the MikroTik Cloud Hosted Router (CHR) virtual machine

- Download: https://www.vmware.com/products/workstation-pro.html

- Ensure VMnet8 (NAT) virtual network adapter is available

b) Docker Desktop for Windows

- Purpose: Runs Prometheus, SNMP Exporter, PostgreSQL, and other containers

- Download: https://www.docker.com/products/docker-desktop/

- Ensure Docker Compose is included (comes with Docker Desktop)

c) Node.js (v18+ recommended)

- Purpose: Runs the React+Vite frontend development server

- Download: https://nodejs.org/

- Verify: node --version && npm --version

d) Python 3.10+

- Purpose: Runs the FastAPI backend server

- Download: https://www.python.org/downloads/

- Verify: python --version && pip --version

e) Git

- Purpose: Clone the NetworkWeaver repository

- Download: https://git-scm.com/

f) SSH Client (built into Windows 10/11)

- Purpose: Connect to MikroTik router for configuration

- Verify: ssh -V

1.2 NETWORK REQUIREMENTS

------------------------

- VMnet8 (NAT) network configured with subnet 192.168.247.0/24

- Host machine must have connectivity to the VMnet8 NAT network

- Ports required: 8000 (backend), 5173 (frontend), 9090 (Prometheus),

9116 (SNMP Exporter), 5432 (PostgreSQL), 8728 (MikroTik API),

161/UDP (SNMP)

1.3 PROJECT STRUCTURE

---------------------

NetworkWeaver/

NetworkWeaver/    docker-compose.yml             monitoring/

Prometheus/

Prometheus.yml       Prometheus scraping configuration

app/

main.py                  FastAPI backend application

frontend/                  React+Vite frontend pplication

package.json

src/

App.jsx

components/

SECTION 2: SETTING UP THE VMWARE CHR ROUTER

2.1 DOWNLOAD MIKROTIK CHR IMAGE

a) Go to https://mikrotik.com/download

b) Download the Cloud Hosted Router (CHR) image for VMware (.vmdk format)

c) Select the latest stable version (verified working: RouterOS 7.20.7)

2.2 CREATE VMWARE VIRTUAL MACHINE

a) Open VMware Workstation Pro

b) Create a New Virtual Machine:

- Choose "Custom (advanced)" configuration

- Hardware compatibility: Latest available

- Guest OS: Linux > Other Linux 5.x kernel 64-bit

- Name: MikroTik-CHR

- Processors: 1 CPU, 1 core

- Memory: 256 MB (minimum) or 512 MB (recommended)

- Network: Use VMnet8 (NAT)

- Use the downloaded CHR.vmdk disk image

c) IMPORTANT - Network Adapter Settings:

- Network Adapter: NAT (VMnet8)

- Ensure "Connect at power on" is checked

2.3 CONFIGURE VMNET8 NAT NETWORK

a) In VMware Workstation, go to Edit > Virtual Network Editor

b) Select VMnet8 (NAT)

c) Configure the subnet:

- Subnet IP: 192.168.247.0

- Subnet Mask: 255.255.255.0

d) NAT Settings:

- Gateway IP: 192.168.247.1 (VMware host gateway)

e) Apply and close

2.4 CONFIGURE STATIC IP ON THE CHR ROUTER

a) Power on the MikroTik CHR virtual machine

b) Log in at the console (default: admin / no password on first boot)

c) Set a password when prompted (in this setup: password is "CHR@admin")

d) Assign static IP address to the interface connected to VMnet8:

/ip address add address=192.168.247.2/24 interface=ether5

Note: The interface name may vary (ether1, ether2, etc.). Use

"/interface print" to see available interfaces and identify which

one is connected to the VMnet8 network.

e) Set default gateway (VMware NAT gateway):

/ip route add gateway=192.168.247.1

f) Verify connectivity from the host:

ping 192.168.247.2

g) Enable SSH for remote management:

/ip service enable ssh

h) Test SSH from the host:

ssh admin@192.168.247.2

SECTION 3: CONFIGURING ROUTEROS SNMP

3.1 ENABLE SNMP ON THE ROUTER

SSH into the router and run the following commands:

a) Enable SNMP:

/snmp set enabled=yes contact="Admin" location="VMware_Lab"

b) Configure the SNMP community 'public':

/snmp community set [ find name=public ] addresses=0.0.0.0/0 \

security=none read-access=yes write-access=no

Note: The 'public' community exists by default in RouterOS.

This command ensures it is configured for open read access.

SECTION 4: SETTING UP DOCKER CONTAINERS

4.1 DOCKER-COMPOSE CONFIGURATION

The docker-compose.yml file in NetworkWeaver/defines ll services.

Key services for monitoring:

services:

db:

image: postgres:15-alpine

container_name:networkweaver-db

environment:      POSTGRES_USER: user

POSTGRES_PASSWORD: password

POSTGRES_DB: networkweaver

ports:

- "5432:5432"

volumes:

- postgres_data:var/lib/postgresql/ta

snmp-exporter:

image: prom/snmp-exporter:latest

container_name:networkweaver-snmp-exporter

ports:

- "9116:9116"

restart: unless-stopped

Prometheus:

image: prom/Prometheus:latest

container_name:networkweaver-prometheus

ports:      - "9090:9090"

volumes:

- ./monitoring/Prometheus/Prometheus.yml:etc/prometheus/prometheus.l

restart: unless-stopped

4.2 PROMETHEUS CONFIGURATION (Prometheus.yml)

Create the file: monitoring/Prometheus/Prometheus.yml

global:

scrape_interval: 15s

evaluation_interval: 15s

scrape_configs:

# SNMP Exporter for MikroTik routers

- job_name: 'snmp'

scrape_interval:15s

scrape_timeout:14s

# Static target for VMware Mikrotik CHR on VMnet8

static_configs:      - targets:

- 192.168.247.2

labels:

hostname: VMware-Mikrotik-CHR

device_id:'26'

metrics_path:/snmp

params:

module: [mikrotik]

relabel_configs:      - source_labels:[__address__]

target_label:__param_target

- ource_labels:[__param_target]

target_label:nstance

- target_label:__address__

replacement: snmp-exporter:9116

IMPORTANT NOTES on Prometheus.yml: The "targets" field lists the SNMP device IP (192.168.247.2), NOT the

SNMP Exporter address

- The relabel_configs section rewrites the scrape target so Prometheus

queries the SNMP er (snmp-exporter:9116) with the device IP as

a parameter

- The module "mikrotik" must match an available module in the SNMP

Exporter's snmp.yml configuration

- The device_id label ('26') must match the device ID in the backend

database

- Use Docker service name "snmp-exporter:9116" (not localhost) since

Prometheus runs inside Docker

4.3 START DOCKER CONTAINERS

a) Navigate to the ConfigWeaver Directory:   cd C:\Users \Desktop\NetworkWeaver\NetworkWeaver

b)Start ll containers:

docker-compose up -d

c) Verify containers are running:

docker ps

Expected running containers:

- networkweaver-prometheus   (port 090)

- networkweaver-snmp-exporter (port 116)

- networkweaver-db           (port 432)

- configweaver-backend       (port 000)

- configweaver-frontend      (port 173)

- networkweaver-wireguard    (port 1820/udp)

SECTION 5: SETTING UP THE FASTAPI BACKEND (PORT 8000)

5.1 BACKEND OVERVIEW

The FastAPI backend serves as the API layer between the frontend and

the MikroTik router. It handles:

- Device registration and management (CRUD operations)

- Device connectivity testing

- Configuration template execution

- Security policy deployment

- Communication with PostgreSQL database

5.2 DATABASE CONFIGURATION

The backend connects to PostgreSQL using:

DATABASE_URL=PostgreSQL://user:password@db:5432/networkweaver

Database credentials (from docker-compose.yml)

- User: user

- Password: password

- Database: networkweaver

- Host: db (Docker service name) or localhost:5432 from host

5.3 START THE BACKEND

The backend runs via uvicorn inside Docker:

uvicorn app.main:app --host 0.0.0.0 --port 8000

If running outside Docker:

a) Install Python dependencies:

pip install -r requirements.txt

b) Set environment variables:

set DATABASE_URL=PostgreSQL://user:password@localhost:5432/networkweaver

c) Run the server:

uvicorn app.main:app --host 0.0.0.0 --port 8000 --reload

5.4 REGISTER THE MIKROTIK DEVICE

After the backend is running, register the MikroTik device:

a) Using the API docs (Swagger UI): http://localhost:8000/docs

b) POST to /devices/ with the following JSON body:

{

"name": "MikroTik",

"ip_address": "192.168.247.2",

"api_port" 8728,

"snmp_community": "public",

"username": "admin",

"password": "admin"

}

c) The API will return the created device with an assigned ID (e.g., 26)

d) IMPORTANT: The device_id in Prometheus.yml ust match this ID.

If the returned ID differs from what's in Prometheus.yml,update

the Prometheus.yml abels section and restart Prometheus:

docker-compose restart Prometheus

SECTION 6: SETTING UP THE REACT+VITE FRONTEND (PORT 5173)

6.1 FRONTEND OVERVIEW

The React+Vite rontend rovides:

- System Overview dashboard (device status, active routers)

- Monitoring page (embedded Grafana dashboards)

- Security Policies page (block website access via L7 rules)

- Devices inventory page

- Configurations page with Device Status and Templates sub-tabs

(Configuration Studio with 20+ MikroTik command templates)

- Logs page

6.2 INSTALL FRONTEND DEPENDENCIES

a) Navigate to the frontend directory:

cd C:\Users\Ronald\Desktop\NetworkWeaver\ConfigWeaver\frontend

b) Install Node.js dependencies:

npm install

6.3 START THE FRONTEND DEV SERVER

a) Start the Vite development server:

npm run dev

This starts the server on http://localhost:5173

b) If running via Docker, the frontend container handles this

automatically on port 5173.

QUICK START SUMMARY (TL;DR)

1. Install: VMware Workstation, Docker Desktop, Node.js, Python, Git

2. Set up VMnet8 NAT with subnet 192.168.247.0/24

3. Deploy MikroTik CHR VM with static IP 192.168.247.2/24 on ether5

4. Enable SNMP on router: /snmp set enabled=yes

5. Clone NetworkWeaver repo and navigate to ConfigWeaver/7.Configure rometheus.ml with target 192.168.247.2 and module mikrotik

6. Run: docker-compose up -d

7. Register device via POST http://localhost:8000/devices/

8. Open http://localhost:5173 - Dashboard should show 1 device onlines

Multiple VMs can be set up and their IP addresses can be added to the app for monitoring.

SNMP must be enabled on each VM, and the app's backend needs to be aware of all the IPs.

To add a new VM:

1. Set up the new VM with a static IP on the same network (e.g., 192.168.247.x).

2. Enable SNMP on the VM: /snmp set enabled=yes

3. Register the new device via POST http://localhost:8000/devices/ with the new VM's IP.

4. The app will automatically begin monitoring the new VM alongside existing ones.